DIY Hotspot Project Management

Scope

Hot Spot

- Order Stuff

- Check uSD Card Inventory

- Receive Stuff

- Assemble

- Test

- PDCA

- Play / Learn / Experiment

What You'll Need:

- Raspberry Pi Zero WH (with headers): This compact version of the Raspberry Pi comes with pre-soldered GPIO headers. Adafruit Store (Cost: $16) —> ACTUAL Total: $22.89  RECEIVED

- MMDVM Hotspot Hat + Case + Antenna + Screen Kit: This kit includes everything you need to turn your Raspberry Pi into a digital mode hotspot. AliExpress (Cost: $17)  —> ACTUAL Total: $13.26 RECEIVED

- Pi Zero Power Supply: To ensure your Raspberry Pi has a reliable power source. eBay (Cost: $7) —> ACTUAL Total: $9.84 Ordered and Received From Amazon

- MicroSD Card (8 GB minimum): For installing the Pi-Star operating system. eBay (Cost: $5)  (Check Inventory) Ordered and Received From Amazon

Schedule

Initial Plan

WO 4/14

Planning

Procurement

Start Survey

Plan Hot Spot Schedule

Cost

Advertised:   $45

Predict:  $55

04/26/25

- Frequency Survey In Process

- Assembly Planning

  - Flash Card

  - Assemble

New uSD Card DS 2 dot Validrive Report (PASS – ACCEPT):

Report #1

| test date and time | 4/21/2025 at 2:47 PM |
| --- | --- |
| declared drive size | 63,864,569,856 (63.9GB) |
| validated drive size | 63,864,569,856 (63.9GB) |
| highest valid region | 63,864,569,856 (63.9GB) |
| hub or drive vendor | mass |
| hub or drive product | storage_device |
| serial number | 121220160204 |

performance details

|  | read | write |
| --- | --- | --- |
| samples | 1,152 | 1,152 |
| minimum | 551 | 566 |
| maximum | 16,838 | 31,767 |
| average | 1,451 | 737 |
| median | 1,100 | 666 |
| std dev | 994 | 1,175 |
| variance | 0.685 | 1.593 |
| total time | 1,671,909 | 850,080 |
| percent | 66.29% | 33.71% |

time measurements in microseconds

Setting Up Your Hotspot:

Flashing Pi-Star onto the MicroSD Card

  - Download Rufus: Head to the Rufus website and download the latest version for your Windows PC. Install Rufus by following the on-screen instructions.

2. Download Pi-Star: Visit the Pi-Star website to download the latest Pi-Star image file. Extract the .zip file to a folder on your PC.

MD5 hash of Pi-Star_RPi_V4.2.3_18-Apr-2025.img:

4f3d947fc4d7e1c501fbaa19c2c85816

CertUtil: -hashfile command completed successfully.

Expected Value:  4f3d947fc4d7e1c501fbaa19c2c85816  Pi-Star_RPi_V4.2.3_18-Apr-2025.img

Expected:   4f3d94 7fc4d7 e1c501 fbaa19 c2c85816

Actual:         4f3d94 7fc4d7 e1c501 fbaa19 c2c85816

Good Checksum  Proceed

3. Prepare the MicroSD Card: Insert the MicroSD card into your computer's card reader. Ensure it is recognized and assigned a drive letter.

4. Configure Rufus:

- Open Rufus and select your MicroSD card as the "Device".

- Click "Select" to choose the Pi-Star .img file you extracted.

- Set the "File system" to FAT32 and leave the "Cluster size" on default.

- Optionally, name your Pi-Star card under "New volume label".

5. Start Flashing: Click "Start" in Rufus. Confirm you're okay with erasing the MicroSD card's data and wait for the flashing process to complete. Close Rufus and eject the MicroSD card safely.

===========================================================

Remember, all you need to do, is download the zipped version of the image
that is most suitable for your Pi / Single Board Computer, Unzip the 
download, and then flash the image to your SD card (using your prefered
image writing tool - see links below for some basic instructions), boot the
Pi, wait 30-40 secs and then login to the admin portal in order to finish
the setup your Pi-Star.

here: http://pi-star/admin/

Default Username: pi-star
Default Password: raspberry

For help getting started, see this *EXCELLENT* video by Craig (W1MSG): Here